192524226 – P Rahul Sai

- ANALYSIS DEVELOPMENT AND VERICATION OF 8 BIT & 16 BIT ARITHMETIC OPERATIONS USING ASSEMBLY LANGUAGE PROGRAM

OBJECTIVES

- To develop and verify assembly language programs for performing 8-bit addition and subtraction using the 8085 microprocessor and observe the results stored in memory.

- To develop and verify assembly language programs for performing 16-bit addition and subtraction using the 8085 microprocessor and analyze the results obtained.

- To develop and verify assembly language programs for performing 8-bit multiplication and division using the 8085 microprocessor and check the accuracy of the results.

- To develop and verify assembly language programs for performing 16-bit multiplication and division using the 8085 microprocessor and validate the results obtained through memory locations.

COMPONENTS REQUIRED

- 8085 Simulator OR 8085 Microprocessor

PROCEDURE

- Open the 8085 microprocessor simulator software.

- Write and enter the assembly language programs for 8-bit and 16-bit

addition, subtraction, multiplication, and division.

- Store the input values in the specified memory locations.

- Execute the programs and observe the arithmetic operations performed by the microprocessor.

- Check the results stored in the specified memory locations.

- Verify the results by comparing them with the expected mathematical outputs.

- verify assembly language programs for performing 8-bit addition and subtraction using the 8085 microprocessor and observe the results stored in memory.

8 BIT ADDITION :

MVI C 00h

LDA 4200h

MOV B A

LDA 4201h

ADD B

JNC SKIP

INR C

SKIP: STA 4202h

MOV A C

STA 4203h

HLT

OUTPUT

8 BIT SUBRACTION :

MVI C 00h

LDA 4201h

MOV B A

LDA 4200h

SUB B

JNC SKIP

CMA

INR A

INR C

SKIP: STA 4202h

MOV A C

STA 4203h

HLT

OUTPUT

- verify assembly language programs for performing 8-bit multiplication and division using the 8085 microprocessor and check the accuracy of the results.

8 BIT MULTIPLICATION:

LDA 4201h

MOV E A

LDA 4202h

MOV C A

REPEAT: DAD D

DCR C

JNZ REPEAT

SHLD 4203h

HLT

OUTPUT

8 BIT DIVISION:

LDA 4201h

MOV B A

LDA 4200h

MVI C 00h

AGAIN: CMP B

JC STORE

SUB B

INR C

JMP AGAIN

STORE: STA 4203

MOV A C

STA 4202h

HLT

OUTPUT

- verify assembly language programs for performing 16-bit addition and subtraction using the 8085 microprocessor and analyze the results obtained.

16 BIT ADDITION

LHLD 4200H

XCHG

LHLD 4202H

DAD D

SHLD 4204H

HLT

OUTPUT

16 BIT SUBRACTION

LHLD 4200H

XCHG

LHLD 4202H

MOV A,L

SUB E

MOV L,A

MOV A,H

SBB D

MOV H,A

SHLD 4204H

HLT

OUTPUT

- verify assembly language programs for performing 16-bit multiplication and division using the 8085 microprocessor and validate the results obtained through memory locations.

16 BIT MULTIPLICATION

LHLD 2050H

XCHG

LHLD 2052H

MOV  B,H

MOV  C,L

LXI  H,0000H

SHLD 2056H

MOV  A,B

ORA  C

JZ   STORE

LOOP:   DAD  D

JNC  NEXT

PUSH H

LHLD 2056H

INX  H

SHLD 2056H

POP  H

NEXT:   DCX  B

MOV  A,B

ORA  C

JNZ  LOOP

STORE:  SHLD 2054H

HLT

OUTPUT

16 BIT DIVISION

LHLD 2050H

XCHG

LHLD 2052H

MOV  B,H

MOV  C,L

MOV  A,B

ORA  C

JZ   STOP

LXI  H,0000H

LOOP:   MOV  A,D

CMP  B

JC   DONE

JNZ  SUBT

MOV  A,E

CMP  C

JC   DONE

SUBT:   MOV  A,E

SUB  C

MOV  E,A

MOV  A,D

SBB  B

MOV  D,A

INX  H

JMP  LOOP

DONE:   SHLD 2054H

XCHG

SHLD 2056H

STOP:   HLT

OUTPUT

RESULT

Thus, the 8-bit and 16-bit arithmetic operations, including addition, subtraction, multiplication, and division, were successfully developed, executed, and verified using assembly language programs in the 8085 microprocessor simulator.